Code No.210255

OR

II  B.Tech. I-Semester Examination Nov/Dec 2002
MECHANICS OF FLUIDS AND HYDRAULIC MACHINES

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain what do you mean by capillarity? Cite examples where surface tension affects play a prominent role.

b)	Calculate the capillarity effect in millimeter in a glan tube of 4mm diameter when immersed in   (i) Water and 	(ii)  In mercury.  The temperatures of the liquids is 20oC, and the value of surface tension of water and mercury at 20oC in contact with air respectively 0.07357 N/m and 0.490 N/m. The contact angle for water  = 0oand for mercury  = 130o24’.

2.a)	On what factors does the pressure variation in the vertical direction depends?

b)	A rectangular gate 2m long and 1.5m wide lies in a vertical plane with its center 2.5m below water surface. Calculate magnitude, direction and location of the total force on the gate.

3.a)	Define and distinguish between the following terms:

- Steady and unsteady flows

- Rotational and irrotation flow. Give examples for each.

b)	A stream function is defined by  = x3 – y3, show that the flow can not be a potential flow. Obtain expression for velocity potential.

4.a)	If for 2-D flow the stream function is given by     = 2xy. Calculate the velocity at the point (3,6). Show that the velocity potential exists for this call and deduce

it.

b)	Show how the linear velocity components may be expressed as a function of the partial derivatives of the stream function.

5.a)	Oil flows through a circular pipe and the velocity distribution across the pipe section is given by ט = ט  max      1-r2

ro2

where ro is the pipe radius and ט is the velocity at radius r from the pipe axis.

b)	Obtain an equation for flow rate for an orifice discharging into atmosphere from a closed venel under pressure.

(contd…2)

Code No.210255				-2-					OR

6.a)	Explain why the hydraulic loss in a pipe is influenced by the surface roughness only at higher Reynolds number.

b)	When 290 litres/sec of water flow in a 30cm pipe line, 60.3 Kw are lost in friction in 300m length of pipe. Calculate the head lost, friction factor, shear velocity and shear stress at the pipe wall.

7.a)	What do you understand by the characteristics of turbine? Explain the governing of turbines.

b)	A turbine working under a head of 36m develops 10,000kW when running at 150rpm. Determine the type of turbine.

8.a)	Derive an expression for the work done by a centrifugal pump.

- What do you understand by priming of a centrifugal pump? Why is it necessary.

***	***	***