## OR

Code No: 210454

II B.Tech- I Semester Examination November/December 2002

# ELECTRICAL SCIENCE

(Common to Electronics & Communication Engineering, Chemical Engineering, Metallurgy & Material Technology, Computer Science & Information Technology, Electronics & Instrumentation Engineering, Electronics & Telematics)

Time: 3 hours						                               Max. Marks: 70

Answer any FIVE questions.

All Questions carry equal marks.

---

1 a)	What are the advantages and disadvantages of using large number of poles in a d.c. machine?

b)	An 8-pole lap wound d.c. generator has 120 slots having 4 conductors per slot.  If each conductor can carry 250H and if flux/pole is 0.05 wb, calculate the speed of the generator for giving 240V on open circuit.  If the voltage drops to 220v on full load, find the rated output of the machine.

2 a)	Derive an expression for the torque developed N-M and in kg-m in a dc motor.

b)	Determine the value of torque in kg-m developed by the armature of a 6-pole wave-wound motor having 492 conductors, 30 mwb per pole when the total armature current is 40A.

3 a)	Explain in detail the O.C., and S.C., tests of single phase transformer and their use to find regulation of a Transformer.

b)	Open circuit and short circuit tests on a 5 KVA, 200V/40V, 50 HZ 1-phase Transformer gave the following Tests.

O.C. Test:	200 V		1 A		100 W		(L.V. Side)

S.C. Test:	15   V		10A		 85 W		(H.V. Side)

- Draw the equivalent circuit referred to Primary

- Calculate the approximate regulation of the Transformer of 0.8 P.F. lag and leading.

4 a)	Explain the construction of squirrel-cage, and phase-wound induction motors.

Mention their applications.

b)	A 3-phase induction motor has 0.06 ohm rotor resistance, and 0.3ohm standstill resistance per phase.  Find the additional resistance required in the rotor circuit to make the starting torque equal to the maximum torque of the motor.

5 a)	Explain the principle of operation of one type of moving iron instrument showing how it is suitable for use on DC and AC systems.

b)	The total resistance of moving iron voltmeter is 1000W and the coil has an inductance of 0.765 H.  the instrument is calibrated with a full scale deflection 50V DC.  Calculate the percentage error when the instrument is used on (i) 25 Hz supply (ii) 250 Hz supply, the applied p.d being 50V in each case.

Conted…..2

Code No: 210454				-2-				OR

6 a)	Voltage of V=Vm Sinwt is impressed across R-C series circuit.  Give the expression for current and power.

b)	A tungsten filament bulb rated at 500W, 100V is to be connected to series with a capacitance across 200V, 50Hz supply.  Calculate (i) the value of capacitor such that the voltage and power consumed by the bulb are according to the rating of the bulb (ii) the power factor of the current drawn from the supply and (iii) draw the phasor diagram of circuit.

7 a)	Deduce the relation between line voltage, line current and phase voltages, phase currents in a delta connected network.

b)	A three phase 400V, 50Hz a.c. supply is feeding a three phase delta connected load with each phase having a resistance of 25W an inductance of 0.15H and a capacitance of 120mF in series.  Determine the line current, volt-amp, active power and reactive volt-amp.

8 a)	Derive the expression for force on a current carrying conductor placed in a magnetic field.

b)	Define self and mutual inductance. What is coefficient coupling. Derive the relation between them.

###