Code No.210205 Set No. 1

II-B.Tech. I-Semester Examination Nov/Dec 2002

## ELECTRO MECHANICS – I

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.	Explain clearly about singly excited systems and developed expression for force produced in the system.

2.a)	Explain the effects of armature reaction in a d.c. generator and discuss briefly the methods to minimize these effects.

b)	Give the general procedure, step wise to design armature winding of a d.c. machine. Mention the assumptions made.

3.a)	Derive the E.M.F. equation of a d.c. generator.

b)	A 500 Volts, 4-pole, wave wound, 750-rpm d.c. shunt generator supplies a load of 195 A. The armature has 720 conductors and shunt field resistance is 100. Find the demagnetizing ampere turns per pole, if the brushes are given an actual lead of 10o at this load. Also calculate the extra shunt field turns required to neutralize the demagnetization.

4.a)	Clearly explain the process of building up of EMF in a self excited generator.

b)	Discuss the applications of shunt, series and compound generator considering their load characteristics.

5.a)	Derive the standard torque equation of a dc motor from first principles.

b)	A 200V DC shunt motor with a constant main field drives a load the torque of which varies as the square of the speed. When running at 600 rpm it takes 30A. Find the speed at which it will run and the current it will draw if a 20 resistor is connected in series with armature. Neglect motor losses.

6.	Outline the steps to estimate the efficiency of given two d.c. machines by conducting Hopkinson’s test. Draw schematic diagram to illustrate the method.

7.a)	What is the effect of excitation, speed and load on the losses of a DC machine?

b)	A 230V, DC shunt motor is taking 5A when running light (i.e at no load). The armature resistance (including brushes) is 0.2 and field circuit resistance is 115. For an input current of 72A, calculate the shaft output and efficiency. Also calculate the armature current at which the efficiency is maximum.

(Contd…2)

Code No.210205			-2-				Set No.1

8.a)	Explain with neat diagrams the construction and working of a DC series motor starter.

- A 200V shunt motor has a 5-step starter. The maximum current during starting should not exceed 40A. Armature resistance is 0.5Ω. Find the resistance of different steps.

***	***	***

Code No.210205 Set No. 2

## II-B.Tech. I-Semester Examination Nov/Dec 2002

ELECTRO MECHANICS – I

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	What are the significances of energy and co-energy of Energy conversion system.

- Derive an expression for the magnetic force developed in linear electromagnetic system.

2.a)	Explain what is meant by ‘straight line commutation’, ‘Accelerated’ and ‘Retarded commutation’. Discuss the role of interpoles in improving commutation.

b)	Explain the difference between simplex and multiplex windings used in D.C. machine. What are the uses of these windings?

3.a)	Explain with neat sketches, the difference between a progressive and retrogressive winding. Explain why retrogressive winding is not preferred in lap windings but preferred in wave windings.

b)	A 4-pole d.c. shunt generator supply a current of 143 A. It has 492 conductors on the armature lap connected while delivering full load, the brushes are given in actual lead of 10o. Calculate the magnetizing ampere turns per pole. The field winding is shunt connected and takes 10A. Find the number of extra shunt field turns necessary to neutralize demagnetization.

4.a)	Clearly explain the terms “critical field resistance” and “critical speed”.

b)	A 500v D.C shunt generator is supplying a 20KN load and has an armature resistance of 0.2 ohms, shunt field resistance of 250 ohms. Determine the armature current, induced EMF is the contact brush drop is IV/per brush.

5.a)	What are the different methods of speed control of DC motor? Explain.

b)	A 220 V DC shunt motor draws a no load armature current of 2.5A when running at 1400rpm. Determine its speed when taking an armature current of 60A if armature reaction weakness the flux by 3%.

6.a)	Explain why Swinburne’s test cannot be used to determine the efficiency of d.c. series machines.

b)	Explain how a single Swinburne’s test can be used to predetermine the efficiency of a d.c. machine, when used as  (i) a generator and    (ii) as a motor.

(Contd…2)

# Code No.210205			-2-				Set No.2

7.a)	How are large series machines tested, explain.

b)	The Hopkinson’s test on two DC shunt machines gave the following results for full load. Line voltage 250V, line current 45A excluding field currents; motor armature current 385A; field current 5A and 4A. Calculate the efficiency of each machine. Armature resistance of each machine is 0.015.

8.	A 4-pole series wound fan motor runs normally at 600 rpm on a 250V DC supply taking 20A. The field coils are all connected in series. Estimate the speed and current taken by the motor if the coils are reconnected in two parallel groups of two in series. The load torque increases as the square of the speed. Assume that the flux is proportional to the current and ignore losses.

***	***	***

Code No.210205 Set No. 3

II-B.Tech. I-Semester Examination Nov/Dec 2002

ELECTRO MECHANICS – I

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	What are the advantages of analyzing the energy conversion devices by field energy concept?

b)	Prove that energy and co-energy in a linear magnetic system are given by identical expressions.

2.	Give the relevant details for developing lap winding for a d.c. machine, with 4-poles, 22 slot and 6 coil sides per slot. Draw developed diagram showing brushes?

3.a)	When are dummy coils used and which type of d.c. armature winding these will occur?

b)	Calculate the ampere turns for each commutating pole of an 8-pole generator with 107 slots, each containing 1000 ampere conductors. The interpole air-gap is 1.2cm. The flux density in the air gap is to be 0.32 T. Neglect iron parts and leakage.

4.a)	What are the conditions to be satisfied for parallel operation of shunt generators? What is the necessity of ehudizer bar?

- Clearly explain the conditions under which a shunt generators fails to self excite.

5.a)	Why is a starter necessary for a DC motor? Explain the working of a 3-point starter with the help of a neat diagram.

b)	Develop the general expression for the speed of a motor in terms of supply voltage, armature resistance and flux per pole.

6.	The results of a Hopkinson’s test on two similar D.C. machines are as follows:

Line voltage			:  220V

Motor armature current	:  23A

Motor field current		:  0.3A

Generator armature current	:  20A

Generator field current	:  0.4A

Armature resistance of each machine  :  0.5

Calculate the efficiency of each machine.

(Contd…2)

# Code No.210205			-2-				Set No.3

7.a)	What are the advantages of indirect methods of testing of DC machines over direct method? Explain.

b)	A 220 V long – shunt compound motor has the following resistances. Armature including brushes is 2.4, shunt field - 440 and series field 0.6. When running light the motor takes a current of 1.1 A. Find the overall efficiency when the out put is 5 HP.

8.a)	A 220V series motor runs at 800 rpm when taking a current of 15A. The motor has an armature resistance of 0.3 Ω and series field resistance of 0.2 Ω. Find the resistance to be connected in series with the armature if it has to take the same current at the same voltage at 600 rpm. Assume flux is proportional to current.

- Explain what is meant by commutation.

***	***	***

Set No. 4

Code No.210205

II-B.Tech. I-Semester Examination Nov/Dec 2002

ELECTRO MECHANICS – I

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	State and briefly explain the various phenomenon useful for the electromechanical energy conversion in rotating mechanisms.

b)	Energy conversion devices make use of the magnetic field as a coupling medium rather than an electric field. Discuss.

2.	Draw the winding diagram in radial form for a 4-pole 13-slot simplex wave connected d.c. Generator with commutator having 13 segments. The no.of coil sides per slot is 2. Indicate the position of brushes.

3.a)	Derive from first principles the E.M.F. equation of a d.c. generator.

b)	Explain the importance of series field, interpole and compensating windings in d.c. compound machine.

4.a)	What are the conditions to be satisfied  for parallel operation of compound generators?

b)	A 250v long shunt D.C. compound generator supplies a 10kw load and runs at 500rpm. The armature, series, shunt field resistances are 0.2, 0.05 and 200 respectively. Determine the armature current and induced EMF of the generator. Take contact drop per brush a 1 volt.

5.a)	Why is a starter necessary for a dc motor? What is the difference between 3 point and 4 point starters?

b)	Calculate the values of resistance elements for a 6-stud starter. The maximum current at starting is not to exceed 20A for a 200V shunt motor. The armature resistance is 0.5. What is the value of minimum current?

6.	A retardation test is made on a separately excited etc. machine as a motor. The induced voltage falls from 240V to 225V is 25 seconds on opening the armature circuit and 6 seconds. On suddenly changing the armature connection from supply to a load resistance taking 10A. Find the efficiency of the machine when running as a motor and taking a current of f25A on a supply of 250V. The resistance of its armature is 0.4 and that of its field winding is 250.

(Contd…2)

# Code No.210205			-2-				Set No.4

7.a)	What is regeneration test on DC shunt machines? Explain the importance of this test in detail?

b)	A 250V, 4-pole shunt motor has two circuit armature winding with 500 conductors. The armature circuit resistance is 0.25, field resistance is 125 and the flux per pole is 0.02wb. Armature reaction is neglected. If the motor draws 14A from mains. Find the shaft power, shaft torque and efficiency with rotational losses equal to 300 watts.

8.a)	How does a DC motor automatically adjust input to match the mechanical load on the motor?

b)	Explain the armature reaction in dc shunt motors, indicating also a few remedies to its adverse effects.

***	***	***