Code No.:210804

Set No.

1

II/IV B.Tech. I-Semester Examination November/December, 2002.

INORGANIC CHEMICAL TECHNOLOGY

(Chemical Engineering)

Time: 3 hours								          Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are the various safety measures to be considered for a chemical process industry.

- Explain, batch versus continuous chemical processing.

2.	Explain the purification method of municipal waster water into potable safe water.

3.	What are the various methods used in body preparation from clay?  Distinguish between permeable and non-permeable wares.

4.	What are the ISI specifications of cement?  How do you test the quality of cement?  How do you analyze the Portland cement sample?

5.a)	Write in detail the chemical reactions occur in the furnace of Glass manufacturing with temperatures.

- Describe the manufacturing of caustic soda by using De nora cell with physico-chemical principles.

6.a)	With the help of a flow diagram describe the process of nitric acid manufacture by ammonia oxidation process.  Indicate the chemical reactions involved.

- Briefly explain the processes available for concentration of nitric acid to 95%.

7.	Draw a neat flow diagram depicting the various details for the production of sulphuric acid by the contact process?

8.	Draw a neat diagram and label the fluidized bed reactor of denitration of uranium nitrate?

- - -

Code No.:210804

Set No.

2

II/IV B.Tech. I-Semester Examination November/December, 2002.

INORGANIC CHEMICAL TECHNOLOGY

(Chemical Engineering)

Time: 3 hours								          Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are the unit operations and unit processes, explain with examples.

- What is the nature of working expected from chemical engineers in petroleum refineries.

2.	Explain the scientific methods of treating Tannery wastes from environmental protection point of view.

3.	What is clay?  What is the effect of temperature on clay?  What are the various ingredients and their functions in clay? How plasticity of clay can be increased?

4.	Write informative notes on (a) Important process parameters for the manufacture of good cement clinker.  (b) Reactions in the rotary kiln.  (c) Sequence of operations in the manufacture of Portland cement.  (d) Additives for cement.

5.a)	Write various methods involved in the Fabrication of glass articles.

- Describe the manufacturing of Soda ash by Leblanc  process with chemical reactions.

6.a)	Describe the process of ammonia synthesis with a neat flow diagram.

- Discuss briefly about the major engineering problems in the manufacture of ammonia.

7.	How is sulphur produced from pyrites?  Draw the flow sheet and describe?

8.	From the common salt, describe with flow diagram how hydrochloric acid is produced?

- - -

Code No.:210804

Set No.

3

II/IV B.Tech. I-Semester Examination November/December, 2002.

INORGANIC CHEMICAL TECHNOLOGY

(Chemical Engineering)

Time: 3 hours								          Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the general principles applied in studying an industry.

- Explain about the flowcharts and the pilot plants.

2.	Explain the methods of effluent sewage disposal from municipal waster water.

3.	What are porcelain enamels?  How are they manufactured?  What are the applications?

4.	Discuss the significance of  the following with respect to the manufacture and properties of Portland cement.  (a) Lime saturation factor  (b) Silica & Alumina modulus  (c) Cooling of clinker  (d) Heat of hydration.

5.a)	How the soda ash is manufacturing by solvay’s ammonia soda process with diagram and Chemical equations.

- Explain the following special glasses (i) Coloured glasses (ii) Optical glass (iii) Opal glass (iv) Sodaline gas.

6.a)	What are the important sources of CO2 for commercial production of CO2?  Mention the uses of CO2.

- Describe a typical process for CO2 recovery from fermentation with the help of a flow diagram.

7.	Draw the flow sheet and describe the oxidation and reduction process for the production of sulphur?

8.	Hydrogen and chlorine can be combined in a reactor to produce hydrochloric acid.  Draw a flow diagram and describe the process?

- - -

Code No.:210804

Set No.

4

II/IV B.Tech. I-Semester Examination November/December, 2002.

INORGANIC CHEMICAL TECHNOLOGY

(Chemical Engineering)

Time: 3 hours								          Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What is the role of chemical engineers in chemical process industries?

b)	What do you understand by chemical process control?

2.	Explain anaerobic and wet-air oxidation treatments for the recovery of chemicals in municipal water conditioning processes.

3.	What are ceramics?  Describe the manufacture of white wares.  Why white wares are usually glazed?

4.	Explain briefly about (a) Sorel cement  (b) Blast furnace slag cement  (c) Role of Gypsum  (d) Coloured cement.

5.a)	Describe deacon’s method for the manufacturing of chlorine, with necessary diagrams and equations.

- Describe various methods of processing of minerals.

6.a)	Discuss about the steam-hydrocarbon reforming process for the manufacture of hydrogen.

- What are the various methods for purification of hydrogen?  Explain briefly.

7.	Draw the flow sheet and describe the Frasch process for the production of sulphur?

8.	Discuss in detail the economic aspects of hydrochloric acid?

- - -