Code No. 211901

## Set No

1

II/IV B.Tech. I Semester Examination November/December 2002

SIGNALS AND MODULATION THEORY

(Electronics & Computer Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	Prove convolution in time domain is equal to multiplication in frequency domain.

- Obtain the fawner Trans farm of the following signals.

  - f(t) = sin(wet + )

  - f(t) = e-at. cos bt

  - f(t) = t. cos at

2.  a)	State and prove Parseval’s theorem.

- State and prove the properties of auto correlation function.

- Write short notes on Impulse response of an LTI system.

- Obtain the relationship between auto correction function and power spectral density.

3.  a)	Distinguish between synchronous detection and asynchronous detection.

- Write down the mathematical expression for FM wave.

- Draw the spectrum of an FM wave

- Draw the circuit for direct generation of an FM wave and explain its working.

4.  a)	Distinguish between synchronous and Asynchronous time division Multiplexing.

- With power spectra, explain flat top sampling.

- Describe a method of generation of PDM.

5.	Derive the expression for probability of error in PCM system.

6.	Explain in detail the operation of a coherent binary PSK transmitter and receiver and derive bit error probability.

7.	Write short notes on

(i)  Companding 		(ii)  PAM		(iii)  QAM	(iv)  QPSK

8.	Write short notes on

(i)  Delta Modulation		(ii)  Generation of SSB Signal

(iii) VSB			(iv)  Line Coding Techniques.

@@@@@

Code No. 211901

## Set No

2

II/IV B.Tech. I Semester Examination November/December 2002

SIGNALS AND MODULATION THEORY

(Electronics & Computer Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.	A function x (t) has a Fourier transform

1

X (W) =		   e-2w²/W²+1

W2 + 1

Using Fourier transform properties, write down the Fourier transform of the following:

(a)	x (2t)

(b)	x (t-2) eft

(c)	4 .  dx(t)

dt

(d)	t

∫      x() d

-

2.  a)	Distinguish between energy signal and power signal

- State Parseval’s theorem

- Verify Parseval’s theorem for the following signal.

x (t) = 5 e-2t. u(t)

3.	A system is characterised by the following equation.

n

y (n) =    x (k)

k=

Determine whether it is

a) Linear	b) Causal 	c) Shift Invariant	d) Statile

4.	A single tone modulating signal em=Em cosmt amplitude modulates a carrier

ec = Ec cosct.

- Derive an expression for the modulated wave

- Derive an expression for the modulation index

- Draw the A.M. Wave form and its spectrum.

- Derive the expression for total power saving in an SSB wave.  Compared to A.M. wave (DSBFC).

Contd…..2.

Code No. 211901				::2::				Set No: 2

5.  a)	Discuss the effect of modulation index on bandwidth in Frequency modulation

- A carrier is frequency modulated.  For what values of , all the power doesn’t lie in the side bands.

- Distinguish between narrow band FM and wide band F.M.

6.  a)	Write short notes on the signaling characteristics of the following schemes.

a) NRZ	b) Manchester		c) Bipolar	d) High density Bipolar

- With a neat schematic diagram explain the operation of delta modulation scheme.

7.  a)	State the demerits of delta modulation systems.

- Explain the operation of Adaptive Delta Modulation.

- Discuss about Differential Quantization.

8.  a)	What is meant by an optimum filter? Discuss its applications in the recovery of digital data.

- Write short notes on

  - QPSK

  - QAM

@@@@@

Code No. 211901

## Set No

3

II/IV B.Tech. I Semester Examination November/December 2002

SIGNALS AND MODULATION THEORY

(Electronics & Computer Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	State and prove sampling theorem.

- Write short notes on aliasing

- State Dirichlet’s conditions.

2.	With regard to Fourier series representation justify the following statements with

one example.

- Odd functions have only sine terms

- Even functions have no sine terms

- Functions with half wave symmetry will contain only odd harmonics.

3.	Write short notes on

a) 	Cross-correlation 			b)	Auto correlation

c)	Properties of correlation function	d)	Energy density spectrum

4.  a)	State the need for modulation

- State the advantages of SSB modulation scheme

- Explain in detail Armstrong method of generation of FM

- State Carson’s rule for estimating the Bandwidth of an FM wave.

5.	Write short notes on

- Frequency division multiplexing	ii)	Time Division Multiplexing

iii)	Companding				iv)	Delta Modulation

6.  a)	Explain in detail with the help of a block diagram PCM Transmitter and PCM Receiver.

b)	State the need for companding.

7.  a)	Write short notes on DPCM.

- Distinguish between Flat top sampling and natural sampling.

- Write short notes of DPSK.

8.  a)	Compare the different coherent digital modulation schemes based on band width, SNR, probability of error etc.,

- Write short notes on VSB.

@@@@@

Code No. 211901

## Set No

4

II/IV B.Tech. I Semester Examination November/December 2002

SIGNALS AND MODULATION THEORY

(Electronics & Computer Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	Prove that the following functions are orthogonal on the interval (0,2)

1(t)	2(t)

1						     1

0	       1             2            t		     0	       1           2               t

- Expand the signal x(t) = Sin (t) in terms of this orthogonal set.  Find the integral squared error.

2.	Obtain the Fourier transform of the trapezoidal pulse shown below.

F(t)

A

t

-b         -a			   O			      +a	        +b

Contd….2.

## Code No. 211901				::2::			                 Set No: 4

3.  a)	State whether the signal x(t) = 4 sin (6t + /6) is a power signal or energy signal.  If it is a power signal find the normalized power if not find the energy.

- State whether the following systems are linear or non linear

(i)	y (n) = x2 (n)

(ii)	dy (t)

+ 10y (t) = x (t)

dt

(iii)	y (n) = x(n) . n

(iv)	dy (t)

+ 10y (t) + 5 = x (t)

dt

- State whether the following are causal or non causal

(i)  y(n) = x(n) – x(n-1) 		(ii)	y(n) = x(-n)

(iii) y(n) = n.x(n)			(iv)	y(n) = ax(n+2)

4.  a)	Explain how will you obtain phase Modulation from Frequency Modulation.

- Explain in detail how FM wave is demodulated using a ratio-detector.

5.  a)	Explain in detail the working of an envelope detector

- With a neat diagram explain the method of generation of SSB signal using phase shift method.

6.	Write short notes on

  - Aliasing

  - Cross-talk in PTM systems

  - A method of generating PAM

  - TDM

7.  a)	Explain in detail the operation of delta modulation systems.  State the short comings of DM.   How it is avoided.

- Write short notes on DPCM

8.  a)	Explain in detail the working of a DPSK system

- Compare different coherent digital modulation schemes based on bandwidth, SNR, and Bit error probability.

@@@@@