### OR

Code No.210256

II-B.Tech. I-Semester Examination Nov/Dec 2002

ELECTRO MECHANICS – I

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:70

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	State and briefly explain the various phenomenon useful for the electromechanical energy conversion in rotating mechanisms.

b)	Energy conversion devices make use of the magnetic field as a coupling medium rather than an electric field. Discuss.

2.	Draw the winding diagram in radial form for a 4-pole 13-slot simplex wave connected d.c. Generator with commutator having 13 segments. The no.of coil sides per slot is 2. Indicate the position of brushes.

3.a)	Derive from first principles the E.M.F. equation of a d.c. generator.

b)	Explain the importance of series field, interpole and compensating windings in d.c. compound machine.

4.a)	What are the conditions to be satisfied  for parallel operation of compound generators?

b)	A 250v long shunt D.C. compound generator supplies a 10kw load and runs at 500rpm. The armature, series, shunt field resistances are 0.2, 0.05 and 200 respectively. Determine the armature current and induced EMF of the generator. Take contact drop per brush a 1 volt.

5.a)	Why is a starter necessary for a dc motor? What is the difference between 3 point and 4 point starters?

b)	Calculate the values of resistance elements for a 6-stud starter. The maximum current at starting is not to exceed 20A for a 200V shunt motor. The armature resistance is 0.5. What is the value of minimum current?

6.	A retardation test is made on a separately excited etc. machine as a motor. The induced voltage falls from 240V to 225V is 25 seconds on opening the armature circuit and 6 seconds. On suddenly changing the armature connection from supply to a load resistance taking 10A. Find the efficiency of the machine when running as a motor and taking a current of f25A on a supply of 250V. The resistance of its armature is 0.4 and that of its field winding is 250.

(Contd…2)

# Code No.210256			-2-					OR

7.a)	What is regeneration test on DC shunt machines? Explain the importance of this test in detail?

b)	A 250V, 4-pole shunt motor has two circuit armature winding with 500 conductors. The armature circuit resistance is 0.25, field resistance is 125 and the flux per pole is 0.02wb. Armature reaction is neglected. If the motor draws 14A from mains. Find the shaft power, shaft torque and efficiency with rotational losses equal to 300 watts.

8.a)	How does a DC motor automatically adjust input to match the mechanical load on the motor?

b)	Explain the armature reaction in dc shunt motors, indicating also a few remedies to its adverse effects.

***	***	***