Code No.210851 OR

### II-B.Tech. I-Semester Examination Nov/Dec 2002

# MECHANICS OF SOLIDS

(Common to Chemical Engineering, Metallurgy and Material Technology)

Time:3 hours								Max.Marks:70

Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Derive the expression for the extension of a tapering rod of length ℓ uniformly tapering from a diameter d1 at one end to a diameter d2 at the other end when subjected to an axial tensile force P.

- Define Poisson’s ratio for a given material

E = 2.1 x 105 N/mm2

Modulus of rigidity = 0.8 x 105 N/mm2

Find the Bulk modulus for the material.

2.	A beam of length 10 metres is loaded as shown in the figure.

6kN

6kN

4kN/m

2m	6m		2m

Draw S.F. and B.M. diagrams for the beam. Also determine the points of contraflexure, if any.

3.a)	A floor has to carry a load of 10 KN/m2. If the span of each joist is 5m and each joist is 300 x 100mm,calculate the maximum permissible spacing of the joists if the bending stress is not to exceed 100 N/mm2.

b)	A beam is of square section of side ‘b’. If the permissible bending stress is ‘f’, find the moment of resistance when the beam section is placed such that

i) Two sides are horizontal	(ii) One diagonal is vertical.

Find also the ratio of the flexural strengths of the section in the two positions.

(Contd…2)

## Code No.210851				-2-				OR

4.	A simply supported beam carries a uniformly distributed load of intensity 10 KN/m over the entire span of 4 metres. The cross-section of the beam is a T-section having the dimensions shown in the figure below.

150

20     All dimensions in mm.

180

20

Sketch the variation of shear stress across the depth of the section. Indicate the magnitude and position of maximum shear stress.

5.	A beam of length ‘ℓ’ simply supported at the ends carries a point load ‘W’ at a distance ‘a’ from the left end. Find the deflection under the load.

If ℓ = 10m, a = 4m, W = 100KN. Calculate the deflection under the load.

Take E = 2 x 105 N/mm2 and I = 1 x 109mm4.

6.a)	A cylindrical air receiver for a compressor is 1.5m in internal diameter and made of plates 15mm thick. If the hoop stress is not to exceed 100 N/mm2 and axial stress is not to exceed 60 N/mm2 find the maximum safe air pressure.

b)	Find the thickness of metal necessary for a thick steel cylindrical shell of internal diameter 200mm to withstand an internal pressure of 40 N/mm2. The maximum hoop stress in the section is not to exceed 150 N/mm2.

7.	At a point in a strained material, the intensities of normal stress across two planes at right angles to each other are 100 N/mm2 and 40N/mm2, both tensile, and there is a shear stress of 40 N/mm2 across the planes. Locate the principal planes and evaluate the principal stresses. Also obtain the maximum intensity of shear stress and specify planes.

8.a)	A solid shaft running at 160 rpm has to transmit 100 KW of power. The material must not be stressed beyond 60 N/mm2 and the shaft must not twist more than 1o in a length of 3 metres. Select a suitable diameter for the shaft.

b)	Two shafts of the same material are subjected to the same torque. If the first shaft is of solid circular section and the second shaft is of hollow section whose internal diameter is ²/₃ rd of the external diameter, compare the weights of the two shafts.

***	***	***