Code No: 210202

Set No:

1

II B.tech I Semester Examination November/December 2002

PULSE AND DIGITAL CIRCUITS

(Common to Electrical & Electronics Engineering, Electronics and Communication  Engineering, Electronics and Telematics, Electronics and Computer Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1 a)	A 10-Hz square wave is fed to an amplifier.  Calculate and plot the output waveform under the following conditions.  The lower 3dB frequency is (i) 0.3 Hz, (ii) 3.0 Hz (iii) 30Hz.

b)	Draw the output waveform of an RC High pass circuit with a square wave input under different time constants.  Explain the same.

2	Compute and draw to scale the output wave form for C=50PF, C=75pF, and C=25pF if the input is a 20V step for the following Circuit.

3 a)	Explain in detail about the transistor switching times.

b)	A silicon transistor operates at room temp.  in common emitter configuration with Rc = 5K from a 10 V collector supply.  The transistor has hfe = 50,

r bb’ = 50, Rce = 80 K Rb’c = 4 M. Calculate the input impedance

a)	Just before the transistor enter saturation

b)	Just after the transistor entering into saturation

c)	When the emitter currents is 1.1 times saturation current.

4 a)	Define i) Rise time (ii) Fall time (iii) Storage time (iv) Delay time of  a transistor switch.  Give the expressions for rise time and fall time in terms of transistor parameters and operating currents.

b)	In the emitter-coupled clipper, transistors are silicon npn type.  The magnitudes of the parameters are Vcc = 4.5 V, Vee = -12V Rc = 2.7 K Re = 10 K  VBB2 = 2V. Plot V0 as a function of Vi . Show where clipping levels occur.  What is the total range of Vi over which output responds?

Contd…2

Code No: 210202				.. 2..				Set No: 1

5 a)	Draw a neat circuit diagram of UJT relaxation oscillator and explain its operation.  Can it be used as time-base circuit of a CRO?  If so how and if not why?

b)	Draw a Miller time-base circuit with n-p-n transistors and explain its operation.  Compare its performance with that of UJT relaxation oscillator.

6 a)	Explain the operation of unidirectional diode sampling gate with neat sketch of waveforms.  Illustrate the effect of different levels of control voltage on gate output.

b)	Discuss the advantages and disadvantages.

7 a)	Explain the operation of Astable multivibrator with a circuit diagram and relevant wave forms.

b)	A collector-coupled monostable multi using n-p-n silicon transistors has the following parameters : VCC = 12V, VBB = 3 V, RC = 2K, R1 + R2 = R = 20 K, hFE =30, rbb’ = 200 and C = 1,000 PF. Calculate and plot to scale the waveslapes at each base and collector.  Also, find the width of the output  pulse.

8 a)	Explain the operation of bistable multi with a relevant triggering circuit and waveforms.

b)	Germanium transistors with (hFE)min = 40 are used in the fixed-bias flip-flop with collector catching diodes.  The circuit parameters are Vec = 18V, V=VBB = 6V, RC = 1.5 K, R1 = 5K, and R2 = 25K.  Find the stable state voltages and currents.

####

Code No: 210202

Set No:

2

II B.tech I Semester Examination November/December 2002

PULSE AND DIGITAL CIRCUITS

(Common to Electrical & Electronics Engineering, Electronics and Communication  Engineering, Electronics and Telematics, Electronics and Computer Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1 a)	A Signal v = V e-t/T is applied to a double differentiator.  Find the output if T=R1C1 = R2 C2.  Plot the response.

b)	Write a short notes on High pass RC circuit as a differentiator.

2 a)	A Symmetrical square wave whose peak-to-peak amplitude is 2V and whose average value is Zero is applied to an RC integrating circuit.  The time constant is half the period of the square wave.  Find the peak-to-peak value of the output amplitude.

b)	Write a short note on RC low pass circuit.  Draw the output if a step input is applied.

3 a)	Explain the operation of two level clipper circuit with equations and also draw the transfer characteristics.

b)	The input to the diode differentiator comparator is ramp whose slope is 0.1 V per second.  Reference level is VR = 0.  Amplifier gain 10 and 1 = 2 = 100 micro second.  What is peak-to-peak value of output?

4 a)	Explain about different compensation techniques used for temperature compensation of clippers.

b)	In the circuit given below Rs = Rf = 100, R =  , R = 10 K,C = 1 F, V = 0.  The input is as shown  below.  Draw the output wave form and label all voltages assuming the capacitor is initially uncharged.

Contd…2

Code No: 210202			.. 2..					Set No: 2

5 a)	Draw the circuit of a constant current sweep circuit and derive the expression for sweep voltage.

b)	In transistor constant current sweep circuit what happens if temperature is increased?

What sort of circuit provides temperature compensation?

6 a)	Explain the operation of sampling gate using transistor.

b)	Discuss on balanced choppers.

7 a)	Explain how an Astable multivibrator can be used as an voltage-to frequency converter with a circuit & Waveforms.

b)	Design an Astable multivibrator to generate 100 KHZ signal with 50% duty cycle.  Plot to scale the collector and base waveforms of each transistor.  Given Vcc=12V, hFE = 100, Ic (sat) = 200 A.

8 a)	Explain the operation of mouostable multivibrator with waveforms.

b)	The self-biased flip-flop uses silicon Transistors with (hFE) min = 30.  The circuit parameters are Vec = 6V, Rc = 1K1 R1 = 9.1 K, R2 = 20K1 and Re = 430 .

###

Code No: 210202

Set No:

3

II B.tech I Semester Examination November/December 2002

PULSE AND DIGITAL CIRCUITS

(Common to Electrical & Electronics Engineering, Electronics and Communication  Engineering, Electronics and Telematics, Electronics and Computer Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1 a)	The periodic wave form shown is applied to an RC integrating circuit whose time constant is 10  sec.  Sketch the output.

b)	Calculate the maximum and minimum values of output voltage with respect to ground.

- The input Vi is a 20-V step calculate and plot to scale the output voltage for the following circuit.

3 a)	Draw the clamping circuit with r = 10 K and C = 1 F.  Assume for the diode rf = 100, rr = , and VBE = 0.6 V.A symmetrical square wave of amplitude 10V, extending from+ 5V to -5V and frequency of 5KHz is applied.  Draw the output for the first three cycles.

b)	What is a slicer? Explain with a circuit diagram.

4 a)	Explain how a transistor can be used as a switch.  Under what conditions a transistor is said to be “OFF” and “ON” respectively?

b)	Explain the phenomenon of “latching” in a transistor switch.

c)	Explain about switching times of a transistor.

Contd…2

Code No: 210202			.. 2..					Set No: 3

5 a)	Draw a bootstrap circuit using a Darlington circuit.  What are its merits and limitations?

b)	Discuss about the recovery time of a sweep circuit.  How do you achieve short recovery time?

6	What is meant by blocking oscillator?  Explain the principle of operation of mono stable blocking oscillator with base timing.  Sketch the current waveforms and derive an expression for current pulse width.  Show the equivalent circuit.

7 a)	Discuss the various methods of triggering a multivibrator.

b)	Explain the operation of Schmitt trigger circuit with wave forms.  Also explain how it can be used as squaring circuit.

8 a)	Explain the operation of bistable multi with a circuit and relevant waveforms

b)	A collector-coupled one-shot using PMP germanium transistors has the following parameters:  VCC = 3V, VBB = 9V, R = 2.7 K, RC = 270, R1 = 1K, R2 = 15 K, C=0.01F, and hFE = 25.  Calculate  and plot waveforms at each base & Collector.

###

Code No: 210202

Set No:

4

II B.tech I Semester Examination November/December 2002

PULSE AND DIGITAL CIRCUITS

(Common to Electrical & Electronics Engineering, Electronics and Communication  Engineering, Electronics and Telematics, Electronics and Computer Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1 a)	Prove that for any periodic input wave form the average level of the steady state output signal form the RC high pass circuit is always Zero.

b)	Prove the above statement for different periodic input waveforms.

2	Derive the different output equations of an RC-low pass circuit for different inputs like

i)	Ramp input

ii)	Square wave input

- Pulse input

- Exponential input

3 a)	Explain the transistor switch in saturation

b)	In the circuit shown below, Rs = Rf = 100, Rr = , R = 10 K, C = 1F, V =0.  Draw the output waveforms and label all the voltages if the input is as shown.

4 a)	Write short notes on practical clamping circuits.

b)	Design diode clamper circuit to restore the positive peaks of i/p signal to the voltage level to 5V.  Assume diode as ideal.

c)	Write short notes on clamping circuit theorem.

5 a)	Distinguish between voltage and current time base generators.

b)	In the current – sweep circuit, how linearity can be corrected through adjustment 	of driving waveform. Illustrate with an example.

6 a)	Explain the triggering arrangement for blocking oscillator.

b)	Discuss the effect of switching due to magnetic saturation show the hysteresis and current waveforms.

Contd…2

Code No: 210202				-2-				Set No: 4

7 a)	Explain the operation of one-shot multi with a triggering circuit & waveforms.

b)	Design an Astable multivibrator to given an output with 100m sec ON period and 50m sec OFF period for Vcc = 15V, hFE = 50, IC(sat) 100A.

8 a)	Distinguish between symmetrical and unsymmetrical triggering methods with the relevant circuits.

b)	Design a bistable multivibrator that gives symmetrical output with Vcc = 12V, VBB = 12V, npn & transistors 1(hFE) min = 20, IC(sat) = 100 mA.  Assume the necessary data.

###