Code No. 210803

## Set No

1

II/IV B.Tech. I Semester Examination November/December 2002

MATERIAL AND ENERGY BALANCE

(Chemical Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.  a)	Write the expressions with notation for partial saturation, relative saturation and percentage saturation.

- Carbon dioxide contains 0.053 mole of water vapor per mole of dry CO2 at a temperature of 35C and total pressure of 750 mm Hg.  Vapour pressure of water vapor at 35C is 44.5 mm Hg.

  - Calculate the relative saturation of the mixture.

  - Calculate the percentage saturation of the mixture.

2.	An air conditioning system taking outside air at 80% humidity cools it to 15C, and reheats the cooled air to 20C to produce conditioned air for a theatre.  Calculate

- kg of water removed per kg of dry air in the cooling unit

- The percentage humidity, relative humidity and the partial pressure of water vapour in the conditioned air.

Data:	Saturation humidity at 30C = 0.027 kg water/ kg dry air.

Wet bulb temperature of original air = 27.2C.

Wet bulb temperature of conditioned air = 16.6C.

Saturation humidity at 20C = 0.0107 kg water / kg dry air.

Vapour pressure of water vapour at 15C = 12.79 mm Hg.

3.  a)	Discuss the usefulness of recycle and purge streams

b)	An aqueous solution containing 33% by weight of Na2SO4 is cooled to 20C when Na2SO4.  10H2O crystallizes out.  Calculate how much of the crystals that would be formed from a 100 kg original solution.  The solubility of Na2SO4 10H2O in water at 20C is 19.4 g/100 g water.

Contd…2.

Code No. 210803			-2-					Set No.1

4.	50 kg of Naphthalene and 200 kg (98% by weight) H2SO4 are charged to pilot

plant reactor.  The reaction was carried out at 433 K for 3 hours when the reaction goes to completion.  The product distribution was found to be 18.6% Monosulphonate Naphthalene (MSN) and 81.4% DiSulphanate Naphthalene (DSN).  The following reactions were observed.

C10H8 + H2SoO4   C10 H7SO3H (MSN) + H2O

C10H7SO3H +H2SO4   C10H6(SO3H)2 + H2O

Calculate i) The quantities of MSN and DSN produced  ii) The complete analysis of the product.

5.  a)	Define heat capacity and specific heat.

b)	Show that Cp = Cv + R

c)	Discuss about the effect of temperature and pressure on the heat capacity of gases.

6.	Calculate the theoretical temperature of combustion of ethane with 25%  excess air.  The average specific heats in kJ/kg.K may be taken as follows:

CO2 = 1.24    O2 = 1.10     Steam = 2.41     N2 = 1.19

The combustion reaction for ethane is

2C2H6(g) + 7O2 (g)  4CO2(g) + 6H2O(g),    Δ H273 = -1560 MJ/k mol.

7.	A natural gas has the following composition, all figures being in volumetric percent:

Methane, CH4 = 80%

Ethane, C2H6 = 15%

Nitrogen,  N2  = 5%

Calculate:

(a) Composition in mole percent (b) composition in weight percent (c) Average molecular weight (d) Density at standard conditions, kg/m3.

8.  a)	State the laws of Dalton and Amagat

b)	A gas mixture contains 0.274 kg mole of HCl, 0.337 kg mole of nitrogen and 0.089 kg mole of oxygen.  Calculate (a) the volume occupied by this mixture and (b) its density in kg/m3 at a pressure of 2000 mm Hg and temperature of 30oC.

@@@@@

Code No. 210803

## Set No

2

II/IV B.Tech. I Semester Examination November/December 2002

MATERIAL AND ENERGY BALANCE

(Chemical Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.a)	Write the principle of wet and dry thermometry.

- A telescope gas holder contain 300m3 of gas saturated with water vapor at 26.5oC and a pressure of 11.2 mm Hg above atmosphere. The barometer reads 720 mm Hg. Vapour pressure of H2O at  26.5oC is 26.2 mm Hg.

2.	Illuminating gas at a temperature of 37.5oC and a pressure of 760mm Hg enters a gas holder carrying 46 grams of water vapor per m3 .  If in the holder, the gas is cooled to 2oC, calculate the weight of water condensed per 1000 m3 of gas entering the holder. The pressure in the holder remains constant.

Vapor pressure of water:

47.5 mm Hg at 37.5oC

5.1 mm Hg at 2oC

3.a)	Define the terms Recycle ratio and Reflux ratio.

- 1000kg of a sodium carbonate solution containing 25% Na2CO3 by weight is subjected to evaporative cooling. During this process 15% of the water present in the solution is evaporated. From the concentrated  solution Na2CO3 . 10 H2O  crystallizes out.  Calculate how much crystals that would be produced if the solubility of Na2CO3. 10 H2O be 21.5 g/100 g water.

4.	A dryer is used to dry 100 kg/hr of wet solids from 20% to 1% moisture by weight by circulating air. The fresh air contains 0.02 kg water vapor per kg dry air is available at a temperature of 303K and 101.325 kPa. The air leaving the dryer is found to contain 0.1 kg water vapor per kg dry air. If a  recycle ratio is maintained at 3 kg dry air in recycle air per kg dry air in fresh air.

Calculate the volumetric flow rate of fresh air assuming the molecular weight of fresh air as a whole as 28.8.

5.a)	Stat the principle behind energy balances and the basis used in establishing a general energy balance for any process.

- Discuss about the separate forms of energy that are included in the general energy balance equation.

Contd…2.

Code No. 210803			::2::					Set No.2

6.  a)	Explain the usefulness of the enthalpy-concentration diagrams.

- Chlorinated diphenyl is heated from 40 to 280oC at the rate of 4000 kg/hr in an indirectly fired heater. The specific heat of the fluid in the temperature range is given by Cp = 0.275 + 0.35 x 10-3 t(kcal/kgoC) where t is in oC. Calculate the heat to be supplied to the fluid in the heater.

7.  	Sulfuric acid can be manufactured by the contact in process according to the following reactions:

S + O2  SO2

2SO2 + O2  2SO3

SO3 + H2O  H2SO4

You are asked as part of the preliminary design of a sulfuric acid plant with a design capacity of 2000 tons/day of 66oBe.  (Baume) [93.2% H2SO4 by weight] to calculate the following:

(a) How many tons of pure sulfur are required per day to run the plant?

(b) How many tons of O2 are required per day?

(c) How many tons of water are required per day?

8.  a)	State Raoult’s law.  What are its limitations?

b)	Estimate the vapour phase composition at 60oC in equilibrium with a liquid mixture containing 40 mole% benzene and 60 mole % toluene.  Also calculate the composition of the liquid mixture, which boils at 90oC and 101.32 k Pa. Vapour pressure data as given as:

Temp., oC		Vapor pressure of 		Vapor pressure of

Benzene, k Pa		    toluene, k Pa

60				51.3				18.7

90			          135.05				54.4

@@@@@

Code No. 210803

## Set No

3

II  B.Tech. I Semester Examination November/December 2002

MATERIAL AND ENERGY BALANCE

(Chemical Engineering)

Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.a)	Define humidity, relative humidity, molar humidity, saturation humidity and percentage humidity.

- By absorption in silica gel you are able to remove 0.93 kg of H2O from moist air at 15oC and 98.6 kPa. The same air measures 1000 m3 at 20oC and 108 kPa when dry. What was the relative humidity of moist air?

2.	In a process benzene is used as a solvent . It is evaporated into dry nitrogen. The resulting mixture at a temperature of 30oC and a pressure of 760 mm Hg has a percentage saturation of 70.  It is desired to condense 80% of the benzene present by a cooling and compressing process. If the temperature is reduced to 0oC, to what pressure must the gas be compressed?

Vapor pressure of benzene :

118.5 mm HKg at 30oC

26.1mm Hg at 0oC

3.a)	What do you mean by the term unsteady states process?

- Two students fares evaluating the average molecular weight of a gas mixture containing Nitrogen and other gases. One of the students using the correct molecular weight of 28 for Nitrogen estimates the average molecular weight as 39.4. The other using an incorrect value of 14 estimates the average molecular weight as 33.8% . Calculate the weight percentage of Nitrogen in the mixture.

4.	Wet solids containing 60% water and 40% Solids (by weight) are to be dried to get solids containing 8% water by weight. Fresh air contains 0.015 kg of water vapor per kg dry air and the air leaving the dryer contains 0.06 kg water vapor per kg . dry air. Part of the exhaust air is recycled back, mixed with fresh air and sent into the dryer. If the quantity of dry air entering the dryer per kg of solids (bone dry is 100 kg calculate.

- The quantity of fresh air

- The fraction of air re-circulated

- The recycle ratio.

Contd….2.

Code No. 210803				::2::				  Set No.3

5.a)	Write the general energy balance equation. Show that heat added is equal to the increase in enthalpy for a non-flow process proceeding at constant pressure and without generation of electrical energy

b)	What is heat balance? Is it different from energy balance? Explain.

- Differentiate between flow and non-flow process.

6.a)	Derive Kirchoff’s equation to calculate heat of reaction at any temperature T.

- Calculate the heat of reaction of the following reaction at 900 K and 1.013 bar pressure:

C7H16  C7 H8 + 4H2

∆Hf data: C7H16 = -224.5 kJ

C7H8 = -50.3 kj

The constants for Cp are as follows:

Gas		   a		 bx103		   cx106

Heptane	29.70		516.80		-162.11

Toluene	  2.41		391.43		-130.70

Hydrogen	29.08		  -0.84		     2.01

7.  a)	Oxygen is prepared according to the equation 2KClO3 = 2KCl + 3O2.  What is the yield of O2 when 9.12 gm of potassium chlorate arre decomposed?  To liberate 2.5 gm of O2  how many grams of chlorate must be decomposed?

- SO2 is produced by reaction:

Cu + 2H2SO4 = CuSO4 + 2H2O + SO2

Calculate (i) the weight of Cu and (ii) the weight of 94% H2SO4 required, to produce 32 gm of SO2

8.  a)	Write about the applications of ideal gas law

- By electrolyzing a mixed brine a mixture of gases is obtained at the cathode having the following composition by weight:

Cl2 = 67%, Br2 – 28%, O2 = 5%

Using the ideal gas law, calculate (i) composition of the gas by volume (ii) Density of the mixture in gm/liter at 25oC and 740 mm Hg pressure (iii) specific gravity of the mixture.

@@@@@

Code No. 210803

## Set No

4

II/IV B.Tech. I Semester Examination November/December 2002

MATERIAL AND ENERGY BALANCE

(Chemical Engineering)

### Time: 3 Hours							Max. Marks: 80

# Answer any Five questions

All questions carry equal marks

---

1.a)	Define saturation, partial saturation, relative saturation and percentage saturation.

- A mixture of acetone vapor and nitrogen contains 14.8% acetone by volume. Calculate the relative saturation and percentage saturation of the mixture at a temperature of 20oC, and the pressure of 745 mm Hg. Vapour pressure of acetone at 20oC is 184.8 mm Hg.

2	Air at 1 atm and 35oC containing 0.053 gm. mole of water vapor per gm. mole of  dry air is to be conditioned at 24oC containing 0.018 gm mole of water vapor per gm. Mole of dry air by cooling part of the air to 10oC containing 0.012 gm. mole of water vapor per gm.  mole of dry air and mixing with uncooled air. The resulting mixture is reheated to 24oC containing 0.018 gm.  mole of water vapor per gm.  mole of dry air.  Calculate  (i) Volume of entering air   (ii) The percentage of dry entering air bypassed.

3.a)	It is desired to make 1000 kg of a mixed acid containing 60% H2SO4, 32% HNO3, and 8% water by blending (i) a spent acid containing 11.3% HNO3, 44.4% H2SO4 and  44.3% water (ii) aqueous 90% HNO3 and (iii) aqueous 98%	H2SO4. All percentages are by weight. Calculate the quantities of each of the three acids required for blending.

- Explain the terms Bypass and Recycle.

4.	An evaporator is fed with 15000 kg/hr of a solution containing 10% Nacl, 15% NaOH and the rest water. In the operation some water is evaporated and NaCl gets precipitated as crystals.  The thick liquor leaving the evaporator contains 45% NaOH, 2% NaCl and the rest water. Calculate  (i) kg/hr of water evaporated  ii) kg/hr of salt precipitated  iii) kg/hr of thick liquor.

5.a)	What is heat of vaporization ? Explain.

- Discuss briefly about the various methods available for calculating heat of vaporization.

Contd…2.

Code No. 210803			::2::					Set No.4

6.a)	Distinguish between heat of combustion and heat of reaction.

- The standard heat of hydrogenation of gaseous propylene (C3H6) to propane is –124 kJ. The heat of combustion of propane is –2.22 MJ at 25oC. Utilizing the known heats of formation of carbon dioxide (-393.6 kJ) and liquid water (-286 kJ), determine the heat of combustion and the standard heat of formation of propylene.

7.	A solution of sodium chloride in water contains 230 gm of NaCl per liter at 20oC.  The density of the solution at this temperature is 1.148 gm/cm3.  Calculate the following items:

(a)  Composition in weight percent (b) volumetric percent of water (Density of water at 20oC = 0.998 gm/cm3)  (c) Composition in mole percent.  (d) Composition in atomic percent (e) Molality (f) gm NaCl/gm H2O.

8.  a)	Write the Claudius – Clapeyron equation.  State the assumptions made in its development.

- Write a brief note on vapor pressure plots.

- The vapor pressure of ethyl ether (molecular weight = 74) is 185 mm Hg at OoC.  The latent heat of vaporization is 92.5 cal/gm at OoC.  Calculate the vapor pressure at 20 and 35oC

@@@@@