Code No. 210854

OR

II/IV B.Tech. I Semester Examination November/December 2002

MATERIAL AND ENERGY BALANCE

(Chemical Engineering)

### Time: 3 Hours							Max. Marks: 70

# Answer any Five questions

All questions carry equal marks

---

1.a)	Define saturation, partial saturation, relative saturation and percentage saturation.

- A mixture of acetone vapor and nitrogen contains 14.8% acetone by volume. Calculate the relative saturation and percentage saturation of the mixture at a temperature of 20oC, and the pressure of 745 mm Hg. Vapour pressure of acetone at 20oC is 184.8 mm Hg.

2	Air at 1 atm and 35oC containing 0.053 gm. mole of water vapor per gm. mole of  dry air is to be conditioned at 24oC containing 0.018 gm mole of water vapor per gm. Mole of dry air by cooling part of the air to 10oC containing 0.012 gm. mole of water vapor per gm.  mole of dry air and mixing with uncooled air. The resulting mixture is reheated to 24oC containing 0.018 gm.  mole of water vapor per gm.  mole of dry air.  Calculate  (i) Volume of entering air   (ii) The percentage of dry entering air bypassed.

3.a)	It is desired to make 1000 kg of a mixed acid containing 60% H2SO4, 32% HNO3, and 8% water by blending (i) a spent acid containing 11.3% HNO3, 44.4% H2SO4 and  44.3% water (ii) aqueous 90% HNO3 and (iii) aqueous 98%	H2SO4. All percentages are by weight. Calculate the quantities of each of the three acids required for blending.

- Explain the terms Bypass and Recycle.

4.	An evaporator is fed with 15000 kg/hr of a solution containing 10% Nacl, 15% NaOH and the rest water. In the operation some water is evaporated and NaCl gets precipitated as crystals.  The thick liquor leaving the evaporator contains 45% NaOH, 2% NaCl and the rest water. Calculate  (i) kg/hr of water evaporated  ii) kg/hr of salt precipitated  iii) kg/hr of thick liquor.

5.a)	What is heat of vaporization ? Explain.

- Discuss briefly about the various methods available for calculating heat of vaporization.

Contd…2.

Code No. 210854			::2::					OR

6.a)	Distinguish between heat of combustion and heat of reaction.

- The standard heat of hydrogenation of gaseous propylene (C3H6) to propane is –124 kJ. The heat of combustion of propane is –2.22 MJ at 25oC. Utilizing the known heats of formation of carbon dioxide (-393.6 kJ) and liquid water (-286 kJ), determine the heat of combustion and the standard heat of formation of propylene.

7.	A solution of sodium chloride in water contains 230 gm of NaCl per liter at 20oC.  The density of the solution at this temperature is 1.148 gm/cm3.  Calculate the following items:

(a)  Composition in weight percent (b) volumetric percent of water (Density of water at 20oC = 0.998 gm/cm3)  (c) Composition in mole percent.  (d) Composition in atomic percent (e) Molality (f) gm NaCl/gm H2O.

8.  a)	Write the Claudius – Clapeyron equation.  State the assumptions made in its development.

- Write a brief note on vapor pressure plots.

- The vapor pressure of ethyl ether (molecular weight = 74) is 185 mm Hg at OoC.  The latent heat of vaporization is 92.5 cal/gm at OoC.  Calculate the vapor pressure at 20 and 35oC

@@@@@