# Code No.310655

## OR

III/IV B.Tech (I Semester) Examination, Nov. 2002.

TECHNIQUES OF METAL JOINING

(Metallurgy and Material Technology)

### Time: 3 Hrs.								Max. Marks: 70

Answer any Five questions

All question carry equal marks

- - -

1.a)	With the help of neat sketches, differentiate between an oxidizing flame, a neutral flame and a reducing flame, in the gas welding processes.

- List the various components of a gas welding unit.

- What are the advantages and disadvantages of using A.C and D.C power supply for manual metal are welding?

2.a)	Describe the MIG and TIG welding processes.

- Compare their relative advantages and disadvantages.

3.a)	Explain the principles and operation of resistance spot welding.

- Discuss the various metallurgical processes that occur during welding of steel, in the weld zone as well as in the base metal.

- Write briefly on safety practices in welding.

4.a)	Describe, with the help of neat sketches, the electron beam welding process.

- Discuss the difficulties to weld dissimilar alloys and how to overcome them.

5.	Write short explanatory notes on any Four of the following:

- Laser welding

- Diffusion welding

- Welding of cast Irons

- Welding of stainless steels

- Welding Electrodes

6.a)	What are the problems associated with welding of aluminium alloys?  How are they overcome?

- Discuss briefly pre and post weld treatments.

7.	Write a critical essay on weld defects, their origin and control.

8.a)	Differentiate between soft solders and hard solders.

- Explain with the help of neat sketch Furnace brazing process.

- What is adhesive bonding process.

***	***	***