Code No.311003

Set No.

## 1

III/IV B.Tech (I Semester) Examination, Nov. 2002.

### INDUSTRIAL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

### Time: 3 Hrs.								Max. Marks: 80

Answer any Five questions

All question carry equal marks

- - -

1 a)	What Comparators? Explain how they are classified?

b)	Describe one method for the measurement of small displacement?

2 a)	Describe the priniciple of operation of a stroboscope?

b)	While measuring the speed of a steam turbine with strobes cope, single line images are observed for stroboscope settings of 3000, 4000 and 5230 r p m.  Calculate the speed of the turbine?

3 a)	Explain the working and construction of a piezoelectric type accelerometer?

- Describe one optical method for the measurement torque?

4	Describe with a neat diagram the working of a two float viscorator.  Describe its merits over the other methods.

5 a)	Define the terms

- Density

- Specific gravity

b)	In an air purge density measuring system the level of liquid in the tank is 100 cms.  An air pressure of 0.15 kg/cm2 is required to balance the liquid column.  Calculate the density and specific gravity of the liquid.

6	What is consistency?  Describe with a neat sketch a sliding cylinder Consistency meter in detail?

7	Explain the principle and working of a capacitance type humidity sensor with the relavent measuring circuits.

8	Write notes on

- Gyroscope

- Freanency Counters

Code No.311003

Set No.

## 2

III/IV B.Tech (I Semester) Examination, Nov. 2002.

### INDUSTRIAL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

### Time: 3 Hrs.								Max. Marks: 80

Answer any Five questions

All question carry equal marks

- - -

1 	Describe any magnetic method of measuring the relative velocity of a system?

2	Discuss how do the vibration analysis can be done by Halography?

3	What are the various methods in estimating the roughness of a surface?  Describe them in detail?

4	Describe how the thermal drying method is used in measuring the relative humidity

5	Describe the working principle of a two float viscorator?

6	Write short notes on

- Chemical methods of measuring humidity

- Frequency Counters.

7	Describe the Units of viscosity specific gravity scales  used in petroleum industries?

8	Derive the merits and demerits of measuring the length and distance by optical methods?

Code No.311003

Set No.

## 3

III/IV B.Tech (I Semester) Examination, Nov. 2002.

### INDUSTRIAL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

### Time: 3 Hrs.								Max. Marks: 80

Answer any Five questions

All question carry equal marks

- - -

1 a)	What are the length standards?  Describe the line and end standards?

b)	Describe the construction and working of  (i) length bars and (ii) Gange blocks.

2 a)	Describe any optical method for the measurement of distance?

b)	A  Vernier  calipers  has an accuracy of 0.02 mm.  The number of divisions crossed on the main scale are 18 while making a measurement.  The 43rd vernier division coincides with the main division.  Calculate the exact length.

3 	Explain the principle and working of a moving coil type transducer for linear velocity measurement.  What are its merits over the other types?

4 a)	Describe the construction and working of  a seismic type vibration transducer.

b)	An accelerometer has a seismic mass of 0.05 kg and a spring constant of 3 x 103 N/M maximum mass displacement is I 0.02 mm. Calculate (a) maximum measurable acceleration (b) natural gravity.

5	Describe in detail with a neat sketch the bubbler tube method for the measurement of liquid density.

6	Explain the principle construction and working of a continuous capillary viscometer with necessary expressions.

7	Define the terms specific humidity and relative humidity.  Describe the construction and working of a wire electrode humidity sensor in detail.

8	Write notes on

- Gamma ray density measuring device

- Wet and dry bulb psychrometer.

Code No.311003

Set No.

## 4

III/IV B.Tech (I Semester) Examination, Nov. 2002.

### INDUSTRIAL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

### Time: 3 Hrs.								Max. Marks: 80

Answer any Five questions

All question carry equal marks

- - -

1 a)	Distinguish between Primay standards, Secondary stands and working stands generally use with respect to measurement system?

b)	What are the drawbacks of optical methods of measurement?

2 a)	Differentiate between tranolatinal & rotational velocity measurement

b)	Explain the principle and operation of Alternating current tachometer?

3 a)	Mention the few uses of accelerometer?

b)	Explain in detail about the optical Techniques for torque measurement?

4	Using a thin circular ring draw a sketch showing arrangement of strain gauge so that vertical and horizontal forces can be measured independent of each other?

5 a)	Distinguish between apparent viscosity and absolute viscosity?

b)	Prove that the capillary and tube viscometers are the most accurate types of viscometer from Different viscosity/shear rate relationship?

6 a)	Describe different techniques for moisture measurement

b)	Explain briefly about

- Electrical hy-gromter

- Psy Chrometer type hy-grometer

7	Define PH value and explain how the glass electrode device is used for the measurement of PH value?

8	Write short notes on

- Oscilloscope

- Sychrometer

- PIEZOELECTRIC METHOD