# Code.No.310452

# OR

III/IV B.Tech. Examination November 2002

CONTROL SYSTEM S

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering and Electronics and Communication Engineering)

time: 3 hours								Max.Marks:70

Answer any Five questions

All questions carry equal Marks

---

1.a)	A sailboat cannot sail directly into the wind and travelling straight downwind is usually slow.  Thus, sailboats tack upwind, i.e., the familiar zig-zag course, and jibe downwind.  A tactician’s decision of when to tack and where to go can determine the artcane of a race.  Describe this process of tacking a sailboat as the wind shifts direction.  Obtain a block diagram of the closed-loop system.

b)	Describe the process of human bio-feedback used to regulate factors such as pain or body temperature.  Is this positive feedback or negative?

2.	The closed-loop transfer function of a control system is given by

G(s) =        0.313 (s+0.8)

(s+0.25)(s2+0.3s+1)

Plot the poles and zeros of this system and discuss the dominance of the complex poles.  What overshoot and rise-time for a unit step input do you expect?

- A feedback system is shown below

R	R	+			Y(s)

G1(s)

k

-  -

S(s+2)

_

S+3

S+0.1

- Determine the steady-state error for a unit step input when k=0.4 and G1(s)=1.

- Select an appropriate G1(s) so that the steady-state error is equal to zero for the unit ramp input.

(contd…2)

# Code.No.310452				-2-			OR

4.	A control system with unity  feedback has a plant

G(s) =   	K(s+2.5)

(s2+2s+2)(s2+4s+5)

sketch the root loans with all pertinent geometrical properties.  Assume k≥0.

5.	Given the loop transfer function

G(s) = 0.164(s+0.2) (-s+0.32)

S2 (s+0.25) (s-0.009)

Obtain the Bode plot and hence the gain and phase margins.

6.	Given the loop transfer function

G(s) = 	      4(s+2)

S(s+1) (s+4)

Obtain the polar plot and assess the stability of the closed-loop system using Nyquist’s criterion.

7.	Consider a unity negative feedback system with the plant

Gp(s) =  	20

S(s+2)(s+3)

We wish to add a lead-lag compensator

Gc (s) = (s+0.15) (s+0.7)

(s+0.015) (s+7)

Show that the gain margin is 24 dB and that the phase margin is 75o for the closed-loop system.

8.	A system is described by its transfer function

G(s) = 		8(s+5)

S3+12s2+44s+48

Determine the state variable representation and hence the unit step response.

***	***	***