# Code.No.3111052

# OR

III/IV B.Tech. Examination November 2002

ELECTROMAGNETIC THEORY

(Common to Electronics and Instrumentation Engineering and Electronics Control Engineering)

Time: 3 hours							Max.Marks:70

Answer any Five questions

All questions carry equal Marks

---

1.a)	Establish the relations between Ē and V for static fields.  Hence evaluate them for a short electric ripole.

b)	For the case of sphere of radius or having a uniform change density Pυ c/m3, find the electric fivx density in different regions and sketch its variation with distance.

2.a)	Establish the relations for the potential and capacitance of a coaxial cable.  Assume suitable parameters.

b)	A 2nC test charge and Q2 located at (0,-4,3) and (0,1,1) respectively.  Find Q2 such that Ē at (0,-3,4) has no Ƴ-component.



3.a)	A circular loop refined by x2+y2=9 carries a d.c. current of 10Amp. In Ø direction.  Determine the resulting field at (0,0,4).

b)	Explain the significance of the term:  Magnetic vector of potential and establish the relations.

4.a)	Define and derive the Maxwell’s curl equations for time varying fields, in both differential and integral forms.



- For a perfect dielectric material with εr=2, and H =2 Cos (106t + x) Z mA/m find

the conduction and displacement current densities, and obtain  D .

5.a)	Define a uniform plane wave.  Obtain the wave equations for a conducting medium.

- Show that a uniform plane wave is a TEM WAVE.

6.a)	Define the terms skindepth, loss tangent and skin-effect resistance.  Evaluate the same for Silver (σ = 6.1 x 107υ/m)  at a frequency at which depth of penetration is 2mm.

b)	For a uniform plane wave normally incident from air on to a perfect conductor, evaluate the two reflection coefficients.  Find the resulting surface current.

(contd…2)

## Code.No.3111052				-2-					OR



7.a)	Given   H =30 Sin (2 Π x 108t-5x)  Z mA/m, for a plane wave, find εr,

λ, υp  and wave impedance in a loss less zedivon.

b)	Distinguish between good conductors and good directories. For fresh water with    εr=81, σ =10-3υ/m find α,β,γ,υp, λ at 1 mHZ.

8.	Write short notes on any Three:

- Pointing sheorem

- Ampere’s  Force Law

- Polarization and different types

- Uniqueness sheorem

***	***	***