Code No. 410452

# OR

IV/IV B.TECH ( I Semester ) Examination November, 2002.

## ELECTRONICS MEASUREMENTS AND INSTRUMENTATION

(Electronics & Communication Engineering)

Time: 3 hours.									Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

-- -

1.a)	Explain terms:

- Significant figures

- Accuracy & Precision

- Confirmity

b)	What kind of error analysis statistical methods . Describe them briefly with examples.

2.a)	What are advantages of DVMs over analog counter parts, explain them by comparing specifications.

b)	Explain with the help of neat diagram

- Dual slope A/D converter

- Successive approximation A/D converter.

3.a)	Draw the block diagram of  Q  meter and  explain and derive formulae used to measure un known inductance and capacitance.

b)	Compute the self capacitance of a coil when the following measurements are made. At frequency f1 = 2 MHz the tuning  capacitance is set at 450 P.F. When the frequency increase to 6 MHz  the capacitance is tuned to 50 P.F.

4.a)	Draw the basic block diagram of a CRO circuit . Explain the function of each block briefly.

- i)  Draw the block diagram of sampling oscilloscope, explain the principle of    working in detail.

ii) What is the need to for a sampling oscilloscope rather than a real time one.

5.a)	Draw the block diagram of a digital phases meter and explain important points in the signal flow.

- What are the advantages of a frequency synthesizer over an oscillator. Describe the construction and mention application.

Contd….2

Code No. 410452					-2-				OR

6.a)	What kind of transducers are used in the measurement of fluid velocity. Describe the construction and explain principle of working.

- Explain the principle of working of thermocouple. Draw characteristics of various thermocouple showing their range of temperatures and output  voltages.

7.a)	Explain the difference between a D.C. bridge and A.C. bridge. Derive expressions for  determining balance conditions in both.

- Describe the construction of CTs and PTs.  Mention their applications.

8.	Write short notes on TWO of the following:

- Magnetic recording techniques

- Distortion  factor meter

- Storage oscilloscope.

- x -