Code No.410201

Set No.

1

## IV /IV B.Tech. I-Semester Examination November 2002

## DIGITAL SIGNAL PROCESSING

(Common to Electrical and Electronics Engineering Computer Science and Information Technology)

Time: 3 hours						                      Max.Marks:80

Answer any FIVE questions

## All questions carry equal Marks

---

1.a)	Find the impulse and step response for the given system:

y(n) + y(n-1) = x(n) –2 x(n-1)

- Test the following systems for linearity, time invariance, causality and stability.

  - y(n) = a|x(n)|

  - y(n) = sin(2 Πbfn/F)x(n)

2.a)	A system is described by the difference equation y(n)-y(n-1)-y(n-2) = x(n-1).

Assuming that the system is initially relaxed, determine its unit sample response h(n).

b)	A system is described by the difference equation y(n) = 3y(n-1) – nx(n) + 4x(n-1) – 2x(n+1) n0.  Verify whether the system is linear, shift in variant and causal. Explain.

3.a)	Consider the first order linear system defined by the difference equation y(n) = ay(n-1) = x(n), where a is the coefficient between Zero and unity.  The input x(n) is restricted to be a periodic sequence with period N i.e. x(n) = x(n+KN) for any integer value of  K. The filter output is assumed to have reached steady state. Determine the writ sample response for a finite impulse response filter which for this class of inputs yields an output y(n) indistinguishable in steady state from the infinite impulse response filter defined by the above difference equation.

b)	Prove   (i)  Re[X(K)] = Re[X(-K)]

(ii)  |X(K)| = +X(-k)| for a real periodic sequence x(n).

- Consider a finite length sequence x(n)  such that x(n) = 0 for n<no and n>N-1 + no.  Compute the samples of Z-transform of x(n) at the point Zk =rej( + 2 / M)k) where  K=0,1,2,--M-1 and M<N.

(contd…2)

### Code No.410201				-2-			Set No.1

5.a)	Explain how the analysis of discrete time invariant system can be obtained using convolution properties of Z transform.

b)	Determine the impulse response of the system described by the difference equation y(n)-3y(n-1)-4y(n-2) = x(n)+2x(n-1).

6.a)	What is an IIR digital filter?

- How are IIR digital filter realized?

- What are the various realizability constraints imposed on transfer function of an IIR digital filter.

7.	Explain the method of determination of pole locations on an ellipse with major axis ‘R’  and minor axis ‘r’ for Chebyshev filter.

8.	Design a low pass filter using rectangular window by taking samples of (n) and with a cut-off frequency of 1.2 radians/sec.

***	***	***

Code No.410201

Set No.

2

## IV /IV B.Tech. I-Semester Examination November 2002

## DIGITAL SIGNAL PROCESSING

(Common to Electrical and Electronics Engineering Computer Science and Information Technology)

Time: 3 hours						                      Max.Marks:80

Answer any FIVE questions

## All questions carry equal Marks

---

1.a)	State and prove frequency shifting property of fourier transform.

- Find the Fourier transform of the following signals

  - x(n) = (n sin won)u(n)	||<1

  - x(n) = (1/4)n u(n+4)

2.a)	A discrete time system can be

i)    Static or dynamic	    (ii) Linear or non linear	(iii) Causal or non causal

iv)  Stable or unstable

Examine the following with respect to the properties above.

y(n) = cos x(n)

y(n) = x(-n+2)

y(n) = x (-n)

- Determine the impulse response and unit step response of the system described by the difference equation.

  - y(n) = 0.6y (n-1)-0.08 y(n-2) + x(n)

  - y(n) = 0.7 y(n-1)-0.1y(n-2)+2x(n)-x(n-2)

3.a)	Let x1(n) = x2(n) = {1   0nN-1

0   otherwise

Find out the circular convolution of these sequence

b)	If x1(n) and x2(n) are two finite sequences, derive the linear convolution of these sequences.

4.	Implement the Decimation in frequency FFT algorithm of N-point DFT where N=8. Also explain the steps involved in this algorithm.

5.	An LTI (linear time invariant system) is described the equation y(n)=x(n)+0.81 x(n-1)-0.81x(n-2)-0.45y(n-2). Determine the transfer function of the system. Sketch the poles and zeroes on the Z-plane.

6.	What is warping effect?  Discuss influence of warping effect on amplitude response and phase response of a derived digital filter from a corresponding analog filter.

(contd…2)

Code No.410201				-2-			Set No.2

7.	Obtain the system functions of normalized butterworth filters for order N=1,2,3 and 4.

8.	Design a high pass filter using hamming window with a cut-off frequency of 1.2 radians/second and N=9.

***	***	***

Code No.410201

Set No.

3

## IV /IV B.Tech. I-Semester Examination November 2002

## DIGITAL SIGNAL PROCESSING

(Common to Electrical and Electronics Engineering Computer Science and Information Technology)

Time: 3 hours						                      Max.Marks:80

Answer any FIVE questions

## All questions carry equal Marks

---

1.a)	A discrete system is given by following difference equation y(n)-5y(n-1=x(n)+4x(n-1)

Where  x(n) is the input and y(n) is the out put. Determine its magnitude and phase response as a function of frequency.

b)	Check the following systems for linearity, causality, time invariance and stability using appropriate tests.

- y(n)=ne|x(n)|      ii)     y(n)=an cos (2 Πn/N)

2.a)	Define a signal. Give some examples of signals. What is signal processing?  Discuss the basic elements of the digital signal processing systems. List their advantageous.

b)	Show that the even and odd parts of a real sequence are, respectively, even and odd sequences.

3.	Consider two periodic sequences x(n) and y(n), x(n) has period N and y(n) has period M. The sequence w(n) is defined as w(n) = x(n) + y(n).

a)	Show that w(n) is periodic with period MN.

b)	Determine W(K) in terms X(K) and Y(K) where X(K), Y(K) and W(K) are the Discrete fourier series coefficients with a period of N,M and MN respectively.

4.	Explain the inverse FFT algorithm to compute inverse DFT of a N=8. Draw the flow graph for the same.

5.	Define stable system and unstable and test the stability of the first-order IIR filter governed by the equation y(n)=x(n)+bx(n-1), where |b|<1.

6.a)	Determine the unit sample response of the ideal low pass filter, why it is not realizable.

- Compare the impulse invariance and bilinear transformation methods.

7.	Discuss the method of determination of Chebyshev polynomials and its properties.

8.	Design a band pass filter to pass frequencies in the range 1-2 radians/second using hanning window N=5.

***	***	***

Code No.410201

Set No.

4

## IV /IV B.Tech. I-Semester Examination November 2002

## DIGITAL SIGNAL PROCESSING

(Common to Electrical and Electronics Engineering Computer Science and Information Technology)

Time: 3 hours						                      Max.Marks:80

Answer any Five questions

## All questions carry equal Marks

---

1.a)	Show that a relaxed linear time invariant system causing if and only if h(n)=n<0.

b)	Determine the range of values of parameter ‘a’ for the linear time invariant system with impulse response.

h(n)=          an  n>0 even

- otherwise

2.a)	Show that the frequency response of a discrete system is a periodic function of frequency.

b)	Obtain the frequency response of the first order system with difference equation

y(0)=x(n)+10y(n-1) with initial condition y(-1)=0 and sketch it.

Comment about its stability.

- Impulse response completely specifies a LSI system. Explain.

3.	Compute the discrete fourier transform of each of the following finite length sequences considered to be of length N.

- x(n) = (n)

- x(n)= (n-no) where 0<no<N

- x(n)=an, 0 ≤ n ≤ N-1

4.	Illustrate the decimation in time procedure for N not a power of 2. Compute it for N=6 (=3.2) also explain the procedure.

5.	Explain the geometrical construction method to determine magnitude and phase response of a digital system.

6.	Discuss magnitude characteristics of an analog Butterworth filter and give its pole locations.

7.a)	Why Chebyshev filter approximation is called Equiripple approximation.

- How the Chebyshev polynomials determined explain.

8.	Design a Finite Impulse Response low pass filter with a cut-off frequency of 1 kHz and sampling rate of 4 kHz with eleven samples using Fourier series.

***	***	***