# Code.No.410354

# OR

IV/IV B.Tech. I-Semester Examination November 2002

PRODUCTION TECHNOLOGY - III

(Common to Mechanical Engineering and Production Engineering and Mechanical Manufacturing Engineering)

time: 3 hours								Max.Marks:70

Answer any Five questions

All questions carry equal Marks

---

1.a)	State three reasons for adopting a system of limits and fits.

- Define three principal classifications of fit with suitable sketches.

- Explain the concept of selective assembly.

2.a)	Compare line and end standards.

- Sketch a Bevel protractor and explain its uses.

- With the help of slips, rollers how do you determine taper angle of a taper plug gauge?  Sketch the set up and explain the procedure.

3.a)	Describe with a neat sketch the working of Autocollimator.

- Find the maximum straightness error for the first five stations along a straight edge given the following autocollimator readings.  Draw a graph to illustrate the result

Position		Auto collimator  readings  (seconds of arc)

0					-

  - +5

  - -8

  - -12

  - +10

  - +10

4.a)	Define following with reference to surface finish assessment .

i)  Roughness	(ii)  Waviness		(iii)  Lay		(iv)  Sampling length

b)	A rectilinear pen recording of a diamond turned surface is shown in figure.  The sampling length L used was 0.8mm and V/H magnification ratio was 5000/100.  Calculate the parameter Ra if the areas in square millimeters above and below the mean line are as follows:

A		B		C		D		E		F

60		115		96		92		109		70

# Code No.410354				-2-			OR

A		 B	C

D	E	F

D

5.a)	Discuss Taylor’s principle of  Limit gauging.

- With the help of a neat sketch explain working of a Tool maker’s microscope.

6.a)	What is a  Comparator?  Explain about one mechanical comparator with a neat sketch.

- Explain constant chord method of assessing Gear tooth thickness.

7.a)	Define effective diameter of a screw thread.  What modifications must be made on a conventional micrometer to measure effective diameter.

- Explain with a diagram, the test for pitch accuracy of Lead screw of a Lathe.

- List various devices used to carry out alignment tests on machine tools.

8.	Write short notes on any three of the following:

- Angle slip gauges

- Optical flat

- Profilograph

- Thread gauges

***	***	***