Code No.411057

## OR

IV/IV B.Tech (I Semester) Examination, Nov. 2002.

#### POWER PLANT INSTRUMENTATION

(Electronics Instrumentation Engineering

### Time: 3 Hrs.								Max. Marks: 70

Answer any Five questions

All question carry equal marks

- - -

1.a)	What is the basic difference between a Thermal power plant and a Nuclear power plant?

- Explain briefly the non-conventional power generation methods.

2.a)	List the important parameters that are to be measured in a thermal power plant.

- Mention the places where these measurements are to be made. Explain in detail.

3.a)	What is the result if excessive air is supplied to the furnace. How to identify the excessive air supply to the furnace.

- How the fuel efficiency is controlled in a furnace?

4.	How much temperature measurement points that a boiler generally contains? Describe in detail a system that monitors all these points.

5.a)	Mention the sources of vibration in a turbine. How the vibration is measured and controlled?

- Explain what happens if the turbine vibrations exceed the tolerance limits.

6.	Explain the necessity of cooling of generator. Describe the method of hydrogen coolent purity measurement in detail.

7.a)	What is necessity of pH and conductivity measurement for feed water?

- Describe a system for the measurement and control of conductivity of feed water.

8.	Write short notes on the following:

a)   Radiation Detectors      b)   Deaerator level control   c)  Computers in power plant    d)  Paramagnetic Oxygen analyzer.

- x -